LiveMix AI

Science & Formula Reference Document

The Sound Engineer Project  ·  Version 1.0  ·  2026

# 1. The Audio Frequency Spectrum

The human audible range spans 20 Hz to 20,000 Hz. Every instrument and voice lives within this spectrum. Understanding which zone each instrument occupies is the scientific basis for all EQ decisions in LiveMix AI.

The 8 Standard Frequency Zones

| Zone | Range (Hz) | What lives here |
| --- | --- | --- |
| Sub Bass | 20 – 60 Hz | Kick drum power, bass synth rumble, felt more than heard |
| Bass | 60 – 250 Hz | Bass guitar fundamentals (41–250 Hz), kick body, bass signals peak at 90–200 Hz |
| Low Mids | 250 – 500 Hz | Warmth zone — fundamentals of most acoustic instruments, mud accumulates here |
| Mids | 500 – 2,000 Hz | Character zone — vocals, guitars; too much causes honky/hollow sound |
| Upper Mids | 2,000 – 4,000 Hz | Presence zone — most sensitive human hearing range; vocals intelligibility |
| Presence | 4,000 – 6,000 Hz | Clarity and attack of percussive instruments; brightness of guitars |
| Brilliance | 6,000 – 12,000 Hz | Air and sparkle; cymbal shimmer, vocal air |
| Air | 12,000 – 20,000 Hz | Ultra-high shimmer; excessive boost adds hiss and noise |

📌 Source: AES + SoundGym + Gear4Music Audio Frequency Reference (2023–2025)

# 2. Instrument Frequency Profiles — Scientific Basis

Each instrument has a fundamental frequency range (where its notes live) and a harmonic range (overtones that extend beyond). The values below are sourced from AES standards, ResearchGate musical acoustics papers, and established mixing references.

## 2.1 Bass Guitar

| Parameter | Value & Justification |
| --- | --- |
| Fundamental Range | 40 – 300 Hz ¦ Lowest open string E1 = 41 Hz, highest fret ~300 Hz |
| Harmonic Extension | Up to 2,000 Hz ¦ String clarity and finger noise lives here |
| ideal_range_db | -28 to -16 dBFS ¦ Must sit below vocals and guitars in the mix |
| clip_threshold | -6 dBFS ¦ AES live sound headroom standard (6–12 dB below 0) |
| q_cut (surgical) | 1.4 – 2.0 ¦ Narrow cut for mud removal at 250–400 Hz |
| q_boost (musical) | 0.7 ¦ Wide boost for warmth at 80–120 Hz |
| Key Problem Zone | 250 – 400 Hz ¦ Clashes with kick drum body and low-mid of guitars |
| High-Pass Filter | Apply at 40 Hz ¦ Remove sub-rumble below bass guitar's lowest note |

Mud check: if energy(250–400 Hz) > energy(60–100 Hz) + 6 dB → cut 280–320 Hz by 3–5 dB, Q=1.4

## 2.2 Electric Guitar (Lead & Rhythm)

| Parameter | Value & Justification |
| --- | --- |
| Fundamental Range | 80 – 1,200 Hz ¦ Low E2 = 82 Hz, high frets ~1.2 kHz |
| Harmonic Extension | Up to 6,000 Hz ¦ Presence and bite of the guitar tone |
| ideal_range_db | -28 to -18 dBFS ¦ Should sit below or equal to vocals |
| clip_threshold | -6 dBFS |
| q_cut | 1.3 – 1.8 ¦ Narrow cut for harshness at 3–5 kHz |
| q_boost | 0.7 ¦ Wide boost for presence at 2–3 kHz |
| Key Problem Zone | 800 Hz – 1.2 kHz ¦ Boxy honky zone; clashes with vocals |
| High-Pass Filter | Apply at 80 Hz ¦ Clean low-end, leave room for bass guitar |

Harshness check: if energy(3,000–5,000 Hz) > energy(1,000–2,000 Hz) + 8 dB → cut 3.5 kHz by 2–3 dB, Q=1.5

## 2.3 Vocals

| Parameter | Value & Justification |
| --- | --- |
| Fundamental Range | 80 – 1,100 Hz ¦ Male chest: 85–180 Hz, female: 165–1,000 Hz |
| Presence & Intelligibility | 2,000 – 5,000 Hz ¦ Where vocal clarity comes from, not fundamentals |
| Sibilance Zone | 5,000 – 8,000 Hz ¦ Harsh 's' and 't' sounds, de-essing target |
| ideal_range_db | -24 to -14 dBFS ¦ Vocals should sit on top of the mix |
| clip_threshold | -6 dBFS |
| q_cut | 1.2 – 1.8 ¦ Cut mud at 200–500 Hz with moderate Q |
| q_boost | 0.6 – 0.8 ¦ Wide boost for presence at 2–3 kHz |
| Key Problem Zone | 200 – 500 Hz ¦ Proximity effect and room rumble accumulates here |
| High-Pass Filter | Apply at 80–100 Hz ¦ Standard practice for proximity effect cleanup |

Intelligibility check: if energy(2,000–5,000 Hz) < energy(500–1,000 Hz) - 10 dB → boost 2.5–3.5 kHz by 2 dB, Q=0.7

## 2.4 Drums

| Parameter | Value & Justification |
| --- | --- |
| Full Range | 20 – 20,000 Hz ¦ Kick at 20–100 Hz, snare at 200–400 Hz, cymbals to 20 kHz |
| Kick Punch Zone | 60 – 100 Hz ¦ Where kick drum power and body lives |
| Snare Body | 200 – 400 Hz ¦ Snare fundamental; clashes with bass guitar low-mids |
| Cymbal Attack | 2,000 – 8,000 Hz ¦ Hi-hat and cymbal presence |
| ideal_range_db | -22 to -12 dBFS ¦ Drums are transient-heavy, need more headroom |
| clip_threshold | -3 dBFS ¦ Transient peaks are very fast; need extra headroom |
| q_cut | 1.0 – 2.0 ¦ Snare ring removal needs surgical narrow Q |
| q_boost | 0.7 ¦ Kick boost is always wide |
| Key Problem Zone 1 | 60 – 100 Hz ¦ Kick clashes with bass guitar fundamental |
| Key Problem Zone 2 | 200 – 400 Hz ¦ Snare clashes with bass guitar low-mids |

Kick vs Bass: boost kick @ 80 Hz, cut bass @ same freq → boost bass @ 100–120 Hz where kick is not

## 2.5 Piano / Keyboard

| Parameter | Value & Justification |
| --- | --- |
| Fundamental Range | 27 – 4,186 Hz ¦ Exact piano range: A0 (27.5 Hz) to C8 (4,186 Hz) |
| Harmonic Extension | Up to 12,000 Hz ¦ Overtones extend far beyond fundamentals |
| ideal_range_db | -26 to -16 dBFS ¦ Typically sits behind vocals and lead instruments |
| clip_threshold | -6 dBFS |
| q_cut | 1.1 – 1.5 |
| q_boost | 0.6 |
| Key Problem Zone | 100 – 500 Hz ¦ Overlaps with bass guitar and vocals simultaneously |
| High-Pass Filter | Apply at 80 Hz in band context ¦ Leave sub-low to bass guitar |

## 2.6 Synth

| Parameter | Value & Justification |
| --- | --- |
| Range | 20 – 20,000 Hz ¦ Programmable — can occupy any frequency zone |
| Typical Lead Synth | 500 – 4,000 Hz ¦ Competes directly with vocals and guitar |
| Typical Bass Synth | 20 – 300 Hz ¦ Competes with bass guitar and kick |
| ideal_range_db | -26 to -16 dBFS |
| clip_threshold | -6 dBFS |
| q_cut | 1.2 |
| q_boost | 0.7 |
| Key Problem Zone | 250 – 500 Hz ¦ Universal mud zone for all synth types |

## 2.7 Brass

| Parameter | Value & Justification |
| --- | --- |
| Fundamental Range | 100 – 5,000 Hz ¦ Trumpet: 165–988 Hz fundamental; harmonics to 5 kHz |
| Brightness Zone | 2,000 – 5,000 Hz ¦ Strong harmonics make brass naturally forward in mix |
| ideal_range_db | -26 to -16 dBFS ¦ Brass can easily dominate — needs level automation |
| clip_threshold | -6 dBFS |
| q_cut | 1.4 |
| q_boost | 0.7 |
| Key Problem Zone 1 | 150 – 250 Hz ¦ Mud zone, cut with narrow Q |
| Key Problem Zone 2 | 450 – 550 Hz ¦ Resonance zone, check and cut |
| Fullness Boost | 65 – 95 Hz ¦ Slight boost for warmth if too thin |

## 2.8 Woodwinds

| Parameter | Value & Justification |
| --- | --- |
| Fundamental Range | 100 – 5,000 Hz ¦ Flute: 260 Hz – 2.3 kHz, Clarinet: 150 Hz – 1.5 kHz |
| Breathy Air Quality | Above 8,000 Hz ¦ Characteristic shimmer of flute and oboe |
| Reed Noise Zone | 5,000 – 7,000 Hz ¦ Unwanted reed vibration — check and cut at 6 kHz |
| ideal_range_db | -26 to -16 dBFS |
| clip_threshold | -6 dBFS |
| q_cut | 1.4 ¦ Narrow for reed noise removal |
| q_boost | 0.7 |
| High-Pass Filter | Apply at 60–80 Hz ¦ Remove sub-bass rumble |
| Key Problem Zone | 250 – 500 Hz ¦ Clashes with guitars and brass simultaneously |

## 2.9 Strings (Violin / Cello)

| Parameter | Value & Justification |
| --- | --- |
| Fundamental Range | 80 – 3,000 Hz ¦ Violin G3 = 196 Hz, highest pitch ~3 kHz |
| Harmonic Extension | Up to 12,000 Hz ¦ Rich overtones define string timbre |
| Body Resonance | 200 – 400 Hz ¦ Where the body warmth of the instrument lives |
| Presence Zone | 2,000 – 4,000 Hz ¦ Clarity and bow attack |
| ideal_range_db | -26 to -16 dBFS |
| clip_threshold | -6 dBFS |
| q_cut | 1.5 |
| q_boost | 0.7 |
| Key Problem Zone | 250 – 400 Hz ¦ Clashes with vocals and guitars in the body zone |

# 3. Core Formulas — What LiveMix AI Calculates

## 3.1 FFT — Converting Audio to Frequency Data

The Fast Fourier Transform converts a time-domain audio signal (raw PCM samples) into frequency-domain data (energy at each Hz). This is the core of all our analysis.

FFT output: X[k] = Σ x[n] · e^(-j·2π·k·n/N)   where N = number of samples

Frequency at bin k: f(k) = k · (sample_rate / N)

Magnitude at bin k: |X[k]| = sqrt(real² + imag²)

Energy in dBFS: dBFS = 20 · log10(|X[k]| / N + 1e-9)

📌 We add 1e-9 to avoid log(0). Sample rate = 44,100 Hz. Window size N = 4,096 samples = ~93ms per frame at 44.1kHz.

## 3.2 Peak Frequency Detection — Finding the Exact Problem Hz

After FFT, we find the exact frequency with the highest energy within each instrument's range. This gives us the precise Hz to tell the user.

peak_freq = freqs[ argmax( |X[k]| ) ]   for k where freqs[k] ∈ instrument.freq_range

📌 argmax returns the index of the maximum value. freqs[k] = k × (44100 / N).

## 3.3 Band Energy — Checking Each Frequency Zone

We calculate the RMS (Root Mean Square) energy of each frequency band to detect mud, harshness, and imbalances.

band_rms = sqrt( mean( |X[k]|² ) )   for all k where freqs[k] ∈ [low_hz, high_hz]

band_db  = 20 · log10( band_rms + 1e-9 )

## 3.4 Gain Correction — The Exact dB to Cut or Boost

This is the core quantitative output. We compare current energy to the ideal target and calculate exactly how many dB to cut or boost.

gain_correction_db = current_db - target_db

if gain_correction_db > 0  →  CUT  by abs(gain_correction_db) dB at peak_freq

if gain_correction_db < 0  →  BOOST by abs(gain_correction_db) dB at peak_freq

Example: current_db = -18.3, target_db = -22.5  →  cut = -18.3 - (-22.5) = 4.2 dB

📌 We cap maximum recommended cut at 8 dB and boost at 4 dB. Beyond these values the source recording itself is the problem.

## 3.5 Q Factor — The Width of the EQ Filter

Q (Quality Factor) determines how wide or narrow the EQ filter is around the center frequency. Formula from Sonimus/AES:

Q = center_frequency / bandwidth

bandwidth = upper_freq - lower_freq   (the range of frequencies affected by the filter)

Example: center = 284 Hz, bandwidth = 200 Hz (200–400 Hz)  →  Q = 284 / 200 = 1.42

| Q Value | Use Case |
| --- | --- |
| 0.5 – 0.7 | Wide boost — always use wide Q when boosting (musical, not surgical) |
| 0.7 – 1.5 | General tonal shaping — typical EQ adjustments |
| 1.5 – 2.5 | Moderate surgical cut — removing mud or harshness |
| 2.5 – 4.0 | Narrow surgical cut — removing a specific resonance or ring |
| 4.0+ | Ultra-narrow notch filter — feedback frequency suppression only |

📌 Rule from AES: always cut narrow, boost wide. Cutting with a narrow Q removes the problem without hollowing the sound.

## 3.6 Feedback / Howling Detection — Frame-by-Frame Tracking

Acoustic feedback occurs when a microphone picks up its own output from a speaker, creating a loop. We detect it before it becomes audible by tracking which FFT bins are growing across frames.

growth_rate[k] = ( |X_t[k]| - |X_(t-3)[k]| ) / 3   (dB per frame, over 3 frames ~280ms)

feedback_risk = True   if growth_rate[k] > 1.5 dB/frame for any bin k

feedback_freq = freqs[k]   where growth_rate is maximum

📌 Source: Springer 2025 — Acoustic Feedback Detection paper. Detection threshold of 1.5 dB/frame gives ~200ms warning before audible howl.

## 3.7 GCC-PHAT — Syncing Multiple Phones

When multiple phones stream audio simultaneously, we use GCC-PHAT (Generalized Cross-Correlation with Phase Transform) to find the time delay between each phone and synchronize them.

GCC-PHAT(τ) = IFFT[ X₁(f) · X₂*(f) / |X₁(f) · X₂*(f)| ]

where: X₁(f) = FFT of Phone 1 audio

X₂*(f) = complex conjugate of FFT of Phone 2 audio

Dividing by magnitude = normalization (the PHAT part)

Peak of result = time delay τ between the two phones

📌 Source: DIA System paper, MobiSys 2014. One phone is master — broadcasts WiFi beacon for clock sync. τ tells us spatial position in the venue.

## 3.8 Delay-and-Sum Beamforming — Combining All Phone Signals

Once we know the time delay τ for each phone, we combine all phone signals into one enhanced signal using Delay-and-Sum beamforming.

y(t) = (1/M) · Σ xₘ(t - τₘ)   for m = 1 to M phones

where: M = number of phones

xₘ(t) = audio signal from phone m

τₘ = time delay for phone m (from GCC-PHAT)

y(t) = combined, enhanced audio signal

📌 Source: Advances in Microphone Array Processing, arXiv 2025. Ad-hoc arrays (random placement) still work — phones don't need specific positions.

## 3.9 Utility Score — Rating Each Phone's Audio Quality

Not every phone captures clean audio. Phones in dead corners or noisy spots get a lower utility score and contribute less to the final mix analysis.

SNR = signal_power / noise_power   (Signal-to-Noise Ratio)

utility_score = SNR / (SNR + noise_constant)   where noise_constant = 10

weight = utility_score   (directly used in Delay-and-Sum beamforming)

if utility_score < 0.3  →  flag node as low quality, warn user

📌 Source: Microphone Utility Estimation, Springer 2023. Low utility nodes are down-weighted, not removed — they may still contribute useful spatial info.

# 4. Complete Instrument Profiles — Final Values for instrument_profiles.py

All values below are derived from the formulas and sources above. These are the exact values to use in code.

| Instrument | freq_range (Hz) | ideal_range_db | clip_db | q_cut | q_boost | HP filter (Hz) |
| --- | --- | --- | --- | --- | --- | --- |
| bass_guitar | 40 – 300 | -28 to -16 | -6 | 1.4 | 0.7 | 40 |
| electric_guitar | 82 – 1,200 | -28 to -18 | -6 | 1.5 | 0.7 | 80 |
| vocals | 80 – 1,100 | -24 to -14 | -6 | 1.4 | 0.7 | 80 |
| drums | 20 – 20,000 | -22 to -12 | -3 | 1.8 | 0.7 | None |
| piano | 27 – 4,186 | -26 to -16 | -6 | 1.2 | 0.6 | 80 |
| synth | 20 – 20,000 | -26 to -16 | -6 | 1.2 | 0.7 | 40 |
| brass | 100 – 5,000 | -26 to -16 | -6 | 1.4 | 0.7 | 80 |
| woodwinds | 100 – 5,000 | -26 to -16 | -6 | 1.4 | 0.7 | 60 |
| strings | 80 – 3,000 | -26 to -16 | -6 | 1.5 | 0.7 | 60 |

Problem Zones per Instrument

| Instrument | Problem Zones (Hz) → Action |
| --- | --- |
| bass_guitar | (250–400): mud clash with kick ¦ (60–100): sub-clash with kick fundamental |
| electric_guitar | (800–1,200): boxy/honky zone ¦ (3,000–5,000): harshness/fatigue |
| vocals | (200–500): proximity mud ¦ (5,000–8,000): sibilance / de-ess zone |
| drums | (60–100): kick vs bass clash ¦ (200–400): snare vs bass clash |
| piano | (100–500): clashes with bass + vocals simultaneously |
| synth | (250–500): universal mud zone ¦ depends on patch type |
| brass | (150–250): mud ¦ (450–550): resonance ring |
| woodwinds | (250–500): guitar+brass clash ¦ (5,000–7,000): reed noise |
| strings | (250–400): vocal + guitar clash in body zone |

# 5. Quick Reference — What LiveMix AI Outputs Per Frame

Every 93ms (one FFT frame at N=4096, SR=44100), the system outputs this for each instrument node:

| Output Field | Formula / Source |
| --- | --- |
| peak_freq_hz | argmax(FFT) within instrument freq_range |
| current_db | 20 · log10(band_rms + 1e-9) |
| gain_correction_db | current_db − target_db (negative = cut, positive = boost) |
| q_value | peak_freq / bandwidth (clamped 0.5–4.0) |
| action | 'cut' if gain_correction > 0.5 dB, 'boost' if < -0.5 dB, 'ok' otherwise |
| feedback_risk | True if growth_rate > 1.5 dB/frame over 3 consecutive frames |
| feedback_freq_hz | freqs[k] where growth_rate is maximum |
| utility_score | SNR / (SNR + 10) — how much to trust this phone node |

End of Document  ·  LiveMix AI Science Reference  ·  2026