The Sound Engineer

feedback_detector.py

Complete Science & Implementation Reference

LiveMix AI Project  ·  Phase 1  ·  2026

# Overview — What Is This File?

feedback_detector.py is the early warning system of LiveMix AI. Its only job is to detect acoustic feedback before it becomes audible to the audience — giving the sound manager approximately 200ms to act before the screech happens.

Unlike instrument_profiles.py (static data) and quantitative_analyzer.py (pure math functions), feedback_detector.py is a CLASS with state. It needs to remember FFT frames over time to detect whether frequencies are growing. It watches every active InstrumentNode in the session simultaneously.

| What it is NOT | What it IS |
| --- | --- |
| Not a pure math function | A stateful class that remembers history |
| Not instrument-specific logic | Cross-node — watches ALL members at once |
| Not called once per session | Called every 93ms for every active node |
| Not the same as InstrumentNode.feedback_risk_check() | Dedicated module — more accurate, cross-node aware |

### The 4 Methods

| Method | Input | Output |
| --- | --- | --- |
| __init__ | threshold, window size | Detector ready |
| analyze(node) | One InstrumentNode | Alert dict for that node |
| analyze_all(nodes) | List of all nodes in session | All alerts, sorted by severity |
| suggest_notch(freq_hz) | Frequency in Hz | Exact notch filter values |

# 1. The Science of Acoustic Feedback

## 1.1 What Is Acoustic Feedback?

Acoustic feedback is a phenomenon where a microphone picks up sound from a loudspeaker that is amplifying the same microphone's signal. This creates a closed loop where sound is continuously amplified until it reaches the system's maximum gain — producing the characteristic loud, sustained screech or howl.

The loop looks like this:

Mic → Amplifier → Speaker → Room acoustics → Mic (again) → loop

At most frequencies, the loop gain is less than 1 — meaning each time the signal goes around the loop, it gets slightly weaker. Feedback occurs when the loop gain reaches exactly 1 or above at any frequency — that frequency then grows without limit until the system clips.

## 1.2 Why One Specific Frequency?

Every room has natural resonant frequencies — frequencies where sound reflects and bounces in ways that constructively reinforce each other. These are determined by the room's dimensions, shape, and surface materials. The microphone-to-speaker distance and the gain setting of the amplifier determine which of these resonant frequencies first reaches loop gain of 1.

This is why feedback always starts as a single tone, not broadband noise. The room selects the frequency — the engineer's job is to find it and cut it before it screams.

## 1.3 The Pre-Howl Phase — Our Detection Window

Before feedback becomes audible as a screech, there is a pre-howl phase where the feedback frequency grows rapidly but has not yet reached full amplitude. This is our detection window.

Research from the Springer 2025 acoustic feedback paper established that:

- Pre-howl begins approximately 200–300ms before audible feedback

- During pre-howl, the feedback frequency grows at 1.5–3.0 dB per frame (at 93ms/frame)

- Detecting growth rate > 1.5 dB/frame for 3 consecutive frames gives reliable early warning

- False alarm rate is low because random noise fluctuations do not sustain growth for 3+ frames

| Phase | What is happening |
| --- | --- |
| Normal (t=0) | All frequencies stable, loop gain < 1 |
| Pre-howl start (t=100ms) | One bin starts growing, loop gain approaching 1 |
| Pre-howl sustained (t=200ms) | Same bin growing for 2+ frames |
| Detection (t=280ms) | Growth > 1.5 dB/frame for 3 frames |
| Audible feedback (t=480ms) | Screech begins, loop gain = 1 |

📌  Source: Springer 2025 — Acoustic Feedback Detection via Notch Filter and Frame-Based Tracking

## 1.4 The Growth Rate Formula

The core mathematical operation is measuring how fast the magnitude at each FFT bin is growing across frames. We compare the latest frame to a frame 3 steps back and divide by 3 to get the average growth rate per frame.

growth_rate[k] = (|X_t[k]| - |X_(t-3)[k]|) / 3

Where:

- |X_t[k]| = magnitude of bin k in the latest FFT frame

- |X_(t-3)[k]| = magnitude of bin k three frames ago

- Dividing by 3 gives average growth per frame

- k = the FFT bin index (each bin corresponds to one frequency)

We then find the bin with the maximum growth rate:

max_growth_rate = max(growth_rate[k])  for all k

feedback_freq   = freqs[argmax(growth_rate)]

| growth_rate value | Interpretation |
| --- | --- |
| < 0 | Frequency is getting quieter — no risk |
| 0 to 1.0 dB/frame | Normal fluctuation — monitor only |
| 1.0 to 1.5 dB/frame | Elevated — approaching threshold |
| 1.5 to 2.5 dB/frame | WARNING — feedback likely in 200ms |
| > 2.5 dB/frame | CRITICAL — feedback imminent, act now |

## 1.5 The Notch Filter Solution

When feedback is detected at a specific frequency, the standard solution is a notch filter — a very narrow EQ cut centered exactly on the feedback frequency. The goal is to reduce the loop gain at that frequency below 1, breaking the feedback loop.

Notch filter: cut_db = -12 dB, Q = 4.0, center = feedback_freq

Why -12 dB? Research shows that reducing loop gain by 10-15 dB is sufficient to break most feedback loops without significantly affecting the overall sound quality. -12 dB is the standard starting point.

Why Q = 4.0? A narrow Q (4.0) keeps the notch very tight — only 1/4 octave wide. This minimizes the impact on the overall tone while still cutting the problem frequency hard enough to stop feedback.

| Notch parameter | Value |
| --- | --- |
| cut_db | -12 dB |
| Q | 4.0 |
| center_freq | Detected feedback_freq Hz |

# 2. Class Design — FeedbackDetector

FeedbackDetector is a class because it needs state — it must remember FFT frames over time. Each instance manages detection for an entire band session.

## 2.1 __init__(threshold_db_per_frame, window_frames)

Purpose

Sets up the detector with its two key parameters. These control how sensitive the detector is.

| Parameter | Description |
| --- | --- |
| threshold_db_per_frame | Growth rate above which feedback is flagged. Lower = more sensitive but more false alarms. |
| window_frames | How many frames to compare across. 3 frames = 280ms lookahead. |

What to store

- self.threshold — the dB/frame threshold value

- self.window — number of frames to compare

- self.history — dict mapping node phone_id to list of recent FFT frames

Code

def __init__(self, threshold_db_per_frame=1.5, window_frames=3):

self.threshold = threshold_db_per_frame

self.window    = window_frames

self.history   = {}   # { phone_id: [fft_frame1, fft_frame2, ...] }

## 2.2 analyze(node) → dict

Purpose

The core detection function. Takes one InstrumentNode, looks at its FFT history, calculates the growth rate at every frequency bin, and returns an alert if any bin is growing dangerously fast.

Step by step

- Get the node's fft_history — if fewer than window_frames frames exist, return no risk

- Take the latest FFT frame: latest = node.fft_history[-1]

- Take the frame window steps back: older = node.fft_history[-window]

- Calculate growth rate array: growth = (latest - older) / window

- Find the bin with maximum growth: max_idx = argmax(growth)

- Get the growth rate at that bin: max_rate = growth[max_idx]

- Get the frequency at that bin: feedback_freq = freqs[max_idx]

- If max_rate > threshold → return alert dict

- Otherwise → return no risk dict

One important detail — freqs array

The FeedbackDetector does not run FFT itself — it reads from node.fft_history which was already filled by InstrumentNode.update_frame(). But it needs the freqs array to convert bin indices to Hz. Pass freqs as a parameter to analyze(), or store it from the first call.

Output — alert dict

{

'risk':        True,

'member':      'Sneha',

'instrument':  'electric_guitar_lead',

'freq_hz':     2800.0,

'growth_rate': 2.1,

'severity':    'warning',    # 'warning' or 'critical'

'message':     'Sneha (Electric Guitar Lead) — Feedback risk at 2800.0 Hz! Growth: 2.1 dB/frame'

}

Output — no risk dict

{

'risk':        False,

'member':      'Sneha',

'growth_rate': 0.3

}

Severity levels

| Condition | Severity |
| --- | --- |
| growth_rate > threshold AND <= 2.5 | warning |
| growth_rate > 2.5 | critical |

Code

def analyze(self, node, freqs) -> dict:

if len(node.fft_history) < self.window:

return {'risk': False, 'member': node.name, 'message': 'Not enough frames'}

latest = node.fft_history[-1]

older  = node.fft_history[-self.window]

growth = (latest - older) / self.window

max_idx  = int(np.argmax(growth))

max_rate = float(growth[max_idx])

freq_hz  = round(float(freqs[max_idx]), 1)

if max_rate > self.threshold:

severity = 'critical' if max_rate > 2.5 else 'warning'

return {

'risk':        True,

'member':      node.name,

'instrument':  node.instrument,

'freq_hz':     freq_hz,

'growth_rate': round(max_rate, 2),

'severity':    severity,

'message':     f'{node.name} ({node.instrument.replace("_"," ").title()}) '

f'— Feedback risk at {freq_hz} Hz! Growth: {round(max_rate,2)} dB/frame'

}

return {'risk': False, 'member': node.name, 'growth_rate': round(max_rate, 2)}

## 2.3 analyze_all(nodes, freqs) → list

Purpose

Runs analyze() on every active node in the session simultaneously. Returns a sorted list of all active alerts — most critical first. This is what the master phone dashboard calls every 500ms.

Step by step

- Loop through all nodes

- Call analyze(node, freqs) for each

- Collect results where risk is True

- Sort by growth_rate descending — most dangerous first

- Return sorted alerts list

Code

def analyze_all(self, nodes: list, freqs) -> list:

alerts = []

for node in nodes:

result = self.analyze(node, freqs)

if result['risk']:

alerts.append(result)

alerts.sort(key=lambda x: x['growth_rate'], reverse=True)

return alerts

## 2.4 suggest_notch(freq_hz) → dict

Purpose

When feedback is detected, this returns the exact notch filter parameters to apply. This is the actionable fix — exact numbers the sound manager can apply on their mixer immediately.

Code

def suggest_notch(self, freq_hz: float) -> dict:

return {

'type':       'notch',

'freq_hz':    freq_hz,

'cut_db':     -12.0,

'q':          4.0,

'message':    f'Apply notch filter at {freq_hz} Hz — cut 12 dB, Q=4.0'

}

# 3. How FeedbackDetector Connects to Everything Else

FeedbackDetector sits between InstrumentNode and the master dashboard output. Here is the exact call chain:

Phone mic → WebSocket → main.py → compute_fft()

→ node.update_frame()  [fills node.fft_history]

→ detector.analyze(node, freqs)  [checks growth rate]

→ if risk: detector.suggest_notch(freq_hz)

→ node.get_recommendation()  [includes feedback alert]

→ WebSocket → Master phone dashboard

| File | Role in feedback detection |
| --- | --- |
| instrument_node.py | Stores fft_history — FeedbackDetector reads this |
| quantitative_analyzer.py | compute_fft() produces the FFT data that fills fft_history |
| feedback_detector.py | Reads fft_history, calculates growth, fires alerts |
| session_manager.py | Owns the FeedbackDetector instance, passes all nodes to analyze_all() |
| main.py | Calls session_manager every frame, sends alerts to master phone |

# 4. Complete Code — feedback_detector.py

import numpy as np

class FeedbackDetector:

def __init__(self, threshold_db_per_frame=1.5, window_frames=3):

self.threshold = threshold_db_per_frame

self.window    = window_frames

def analyze(self, node, freqs) -> dict:

if len(node.fft_history) < self.window:

return {'risk': False, 'member': node.name,

'message': 'Not enough frames yet'}

latest = node.fft_history[-1]

older  = node.fft_history[-self.window]

growth = (latest - older) / self.window

max_idx  = int(np.argmax(growth))

max_rate = float(growth[max_idx])

freq_hz  = round(float(freqs[max_idx]), 1)

if max_rate > self.threshold:

severity = 'critical' if max_rate > 2.5 else 'warning'

return {

'risk':        True,

'member':      node.name,

'instrument':  node.instrument,

'freq_hz':     freq_hz,

'growth_rate': round(max_rate, 2),

'severity':    severity,

'message':     f'{node.name} '

f'— Feedback risk at {freq_hz} Hz!'

}

return {'risk': False, 'member': node.name,

'growth_rate': round(max_rate, 2)}

def analyze_all(self, nodes: list, freqs) -> list:

alerts = []

for node in nodes:

result = self.analyze(node, freqs)

if result['risk']:

alerts.append(result)

alerts.sort(key=lambda x: x['growth_rate'], reverse=True)

return alerts

def suggest_notch(self, freq_hz: float) -> dict:

return {

'type':    'notch',

'freq_hz': freq_hz,

'cut_db':  -12.0,

'q':       4.0,

'message': f'Apply notch at {freq_hz} Hz — 12 dB, Q=4.0'

}

# 5. Test Block — What to Write at the Bottom

Simulate two nodes — one with growing frequency (feedback risk), one stable. The detector should fire on the first node only.

if __name__ == '__main__':

from instrument_node import InstrumentNode

import numpy as np

sample_rate = 44100

N = 4096

freqs = np.fft.rfftfreq(N, d=1.0/sample_rate)

# Node 1 — Sneha (Lead Guitar) — simulating feedback at 2800 Hz

node1 = InstrumentNode('Sneha', 'electric_guitar_lead', 'ws_1', 'stage_right')

for i in range(5):

fft_data = np.random.uniform(0.0001, 0.001, len(freqs))

spike_idx = np.argmin(np.abs(freqs - 2800))

fft_data[spike_idx] = 0.01 * (i + 1)   # growing spike

node1.update_frame(fft_data, freqs, -20.0)

# Node 2 — Ravi (Bass) — stable, no feedback

node2 = InstrumentNode('Ravi', 'bass_guitar', 'ws_2', 'stage_left')

for i in range(5):

fft_data = np.random.uniform(0.0001, 0.001, len(freqs))

node2.update_frame(fft_data, freqs, -22.0)

detector = FeedbackDetector(threshold_db_per_frame=1.5, window_frames=3)

print('--- Analyze Sneha (should show risk) ---')

result1 = detector.analyze(node1, freqs)

print(result1)

print('--- Analyze Ravi (should show no risk) ---')

result2 = detector.analyze(node2, freqs)

print(result2)

print('--- Analyze All ---')

all_alerts = detector.analyze_all([node1, node2], freqs)

for alert in all_alerts:

print(' ->', alert['message'])

print('--- Suggest Notch ---')

if all_alerts:

notch = detector.suggest_notch(all_alerts[0]['freq_hz'])

print(notch['message'])

# 6. Quick Reference

| Item | Value |
| --- | --- |
| Detection threshold | 1.5 dB/frame growth rate |
| Critical threshold | 2.5 dB/frame growth rate |
| Window size | 3 frames = 280ms lookback |
| Warning time | ~200ms before audible feedback |
| Notch cut | −12 dB |
| Notch Q | 4.0 (very narrow — surgical) |
| Growth formula | (latest_frame − older_frame) / window |
| False alarm protection | Must sustain growth for 3 frames, not just 1 |
| Source | Springer 2025 — Acoustic Feedback Detection |

End of Document  ·  feedback_detector.py  ·  LiveMix AI  ·  2026